Dynamic Model-Context Alignment (DMCA)

A Closed-Loop Framework for Self-Evolving Digital Twins in Industry 4.0

Kevin Omede  •  MSc Student, Politecnico di Torino × Beihang University  •  kevinomede2002@gmail.com

Problem Statement

Current Digital Twin lifecycle frameworks address DT-to-target-entity alignment through static deployment and periodic manual updates. In heterogeneous edge environments — assets with different CPUs, RAM budgets, and latency SLAs — this approach fails to maintain inference fidelity over time, particularly under concept drift and operational context changes. No existing framework formalizes the trade-off between model accuracy and edge feasibility as a runtime, continuously re-evaluated decision.

Contribution: Five-Stage Closed-Loop Orchestrator

| Stage 1 | DT Profiling — reads hardware & latency constraints from AAS Type 1 JSON (ISO/IEC 30188 data interface layer) |
| --- | --- |
| Stage 2 | Model Selection — TOPSIS multi-criteria ranking: MAE×0.5 + latency×0.3 + params×0.1 + license×0.1; highest-fidelity admissible model |
| Stage 3 | Edge Deployment — quality gate (4 deterministic checks): latency SLA, output shape, validity, memory; writes DeploymentState to AAS |
| Stage 4 | Drift Detection — ADWIN (δ=0.002, Hoeffding bound) + 7-type decision tree; FAR=0.000 validated empirically |
| Stage 5 | Re-alignment — trust-gated operator copilot; SIL-parameterized confirmation gate; atomic swap + audit trail JSONL |

Architectural Position

Situated at the frontier between Cognitive and Agentic DT (Klar et al., 2023; El Maraghy, 2022; Ivanov, 2026), DMCA implements controlled progression toward full DT autonomy. The system acts autonomously under low safety criticality (SIL < 2) while delegating decisions to the operator under high-risk conditions — calibrating trust over time as the primary design objective. Formal mathematical guarantees (Hoeffding bound on FAR, TOPSIS deterministic ranking, hard latency constraints) bound the envelope within which ML models operate.

Key Empirical Results  (GPU NVIDIA T4, validated benchmark)

| Benchmark scope | 8 models × 2 datasets (CMAPSS FD001, ETT-h1) |
| --- | --- |
| Best raw MAE | TimesFM 0.006 — excluded: latency 2039 ms exceeds SLA |
| Selected model | chronos-t5-tiny — MAE 0.174 post-recovery |
| Drift detection | t = 1380 • Recovery t = 1400 • Cycle: 20 timesteps |
| ADWIN threshold | 1.9918 • δ = 0.002 • FAR = 0.000 |
| Drift type accuracy | Overall 80% • Variance shift 100% • Gradual 100% |

Alignment with Open Research Challenges

| Open challenge | DMCA response |
| --- | --- |
| Balance accuracy & feasibility | TOPSIS formalizes the trade-off as runtime multi-criteria optimization |
| Low-latency real-time synchronization | AAS-driven quality gate enforces latency SLA as hard constraint |
| Self-evolving Digital Twin | Closed-loop drift detection → model re-alignment without human intervention |
| Semantic drift (Abbassi, 2026) | Identified future work — requires ontological AAS extension |

Supervisors: Prof. [Relatore Polito]  •  Prof. Yi Li (Beihang)  •  Expected completion: 2026