# DMCA for Smart Manufacturing

Paper Skeleton v0.3 + Phase 2 Lifecycle Loop Design — for IJAMT submission and IRT SystemX joint collaboration

Lead author: Kevin Omede  ·  Politecnico di Torino × Beihang University  ·  Status: v0.3, pre-call SystemX 2026-06-26

# 1 · Manuscript scope

Target journal: International Journal of Advanced Manufacturing Technology (IJAMT, Springer). Target length: 16 pages including references. Submission window: to be aligned with co-authors.

Proposed title: Digital-Twin-Driven Adaptive Model Selection and Drift Recovery for Predictive Maintenance in Smart Manufacturing.

The manuscript repositions the underlying DMCA framework (originally an Industry 4.0 generic adaptive ML loop) on Smart Manufacturing predictive maintenance, anchored on (a) rotating-machinery bearing degradation, (b) AAS-based digital twin standards, and (c) operational coverage constraints on industrial edge devices.

# 2 · Authorship and attribution

| # | Author | Affiliation | Primary section ownership | Role |
| --- | --- | --- | --- | --- |
| 1 | K. Omede | Polito × Beihang | §1, §3, §5, §7 (lead), §8, §9, §11 | Lead author, executor, framework + experiments |
| 2 | Prof. A. Simeone | Politecnico di Torino | §2 (manufacturing PM literature), §7 case study review, §10 | Senior IT, manufacturing framing, IJAMT editorial relationship |
| 3 | Prof. Yi Li | Beihang University | §4 (Digital Twin model, AAS conceptualization), §10 | Senior CN, DT modeling |
| 4 | Dr. A. Belfadel | IRT SystemX | §6 (distributed AAS extension), §2 standards positioning, §10 | Technical collaborator FR, AAS / IEC 63278 expertise |

Note: Prof. N. Anwer (Paris-Saclay/LURPA) introduced the FR collaboration but is not a co-author (out of bandwidth per his communication; never confirmed direct authorship).

# 3 · Manufacturing legitimacy — anchoring argument

Three repositioning moves close the manufacturing gap for IJAMT:

Manufacturing PM economics in §1: cite Aberdeen / Senseye 2024 — general manufacturing unplanned downtime $260k/h, automotive $2.3M/h, Fortune 500 lose 11% of revenue annually. Two-thirds of plants experience monthly unplanned downtime averaging $2M per event. PM is the Industry 4.0 use case zero.

Real PM bearing datasets in §7: PRONOSTIA (FEMTO-ST, PHM 2012 Challenge) and IMS Center (NASA Prognostics Data Repository). CMAPSS retained as aerospace-MRO secondary; ETT-h1 as industrial-energy-infrastructure secondary. Three verticals → multi-asset framework validation.

Manufacturing-aware DT submodel additions in §4: production line criticality tier (A/B/C), IT-OT positioning, Safety Integrity Level (SIL), operator certification level (L1/L2/L3). Closes the manufacturing-context gap of the static Phase 1 AAS.

ISO 10816 vibration severity zones as the bridge from accelerated bench data (PRONOSTIA, minutes) to industrial timing (IMS, days; field bearings, weeks). Standards anchor that scales the empirical results.

# 4 · Manuscript outline and page allocation (16 pages)

| Section | Pages | Owner | Notes |
| --- | --- | --- | --- |
| 1. Introduction | 1.5 | Kevin | Manufacturing PM motivation, economics, contribution map |
| 2. Related Work | 2.0 | Simeone + Belfadel + Kevin | Manufacturing PM, AAS standards, model selection |
| 3. Problem Statement | 0.5 | Kevin | PM admissibility + drift + economic framing |
| 4. Digital Twin Model | 1.5 | Yi Li | AAS-based DT, manufacturing-aware submodels |
| 5. DMCA Framework Architecture | 2.5 | Kevin | 5-stage closed loop |
| 6. Distributed AAS Extension | 1.5 | Belfadel | Cloud/fog/edge submodels, TOPSIS-as-service |
| 7. Manufacturing Case Studies | 2.5 | Simeone + Kevin | PRONOSTIA + IMS missed-alarm simulation, Stage 2 validation |
| 8. Implementation & Validation | 1.0 | Kevin | Drift classifier, design decisions log |
| 9. Results & Analysis | 1.5 | Kevin | Latency cluster, SLA paradox, missed-alarm cascade |
| 10. Discussion | 0.5 | All | Manufacturing implications, standards alignment |
| 11. Conclusion & Future Work | 0.5 | Kevin | Closure + Phase 2 lifecycle loop direction |
| References | 0.5 | All | 30-40 refs |

# 5 · Today's experimental results — Stage 2 validation on manufacturing data

A missed-alarm-cascade simulation was executed today on both target datasets, isolating the temporal-coverage variable (Stage 2 admissibility) from model-accuracy. The simulation assumes ideal detection accuracy on processed monitoring windows: the differentiator is how many windows each model actually serves within the Jetson Nano 50 ms SLA budget.

This is an upper bound on the SLA-violating model — any real model with imperfect accuracy would perform worse, not better. The reported gap is therefore a conservative lower bound on the real operational gap.

## 5.1 · PRONOSTIA — 6 training bearings (real PHM 2012 data)

Setup: σ=3, action window 5 min, baseline = first 10% of trace. Coverage rates from canonical T4 benchmark: PatchTST 85%, Lag-Llama 64%, Moirai-Small 9.3%.

| Bearing | Trace (min) | PatchTST alarms | Lag-Llama alarms | Moirai-S alarms | PatchTST lead (s) | Moirai-S lead (s) | Moirai-S actionable |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bearing1_1 | 467 | 1044 | 523 | 97 | 27780 | 27690 | YES |
| Bearing1_2 | 145 | 45 | 23 | 4 | 440 | 340 | YES |
| Bearing2_1 | 152 | 44 | 22 | 4 | 8770 | 8770 | YES |
| Bearing2_2 | 133 | 563 | 281 | 51 | 5670 | 5540 | YES |
| Bearing3_1 | 86 | 23 | 12 | 3 | 4920 | 4920 | YES |
| Bearing3_2 | 273 | 29 | 14 | 2 | 390 | 190 | NO |

Headline:

Alarm-density ratio mean: 10.4 (range 7.7–14.5 cross-bearing), tightly clustered around coverage-predicted 10.7×.

Bearing3_2 is the safety-critical demonstration: SLA-compliant PatchTST raises first alarm at 390 s pre-failure (actionable, ≥ 300 s action window); SLA-violating Moirai-S raises first alarm at 190 s (not actionable). The framework correctly distinguishes admissible from non-admissible for the asset whose rapid-failure profile leaves no time for late detection.

## 5.2 · IMS Center — Bearing 1 of Test 2 (real NASA data)

Setup: 766 acquisitions × 10-minute interval = 127.7 hours = 5.3 days of realistic industrial timing. Action window: 24 h (industrial PM scheduling). Documented failure: outer race [Lee et al. 2007 dataset notes].

| Model | Alarms | First-alarm lead (h) | Actionable (≥24h) |
| --- | --- | --- | --- |
| PatchTST | 233 | 38.7 | YES |
| Lag-Llama | 116 | 38.5 | YES |
| Moirai-S | 20 | 35.8 | YES |

Headline:

Alarm-density ratio: 11.6 (233/20) — coherent with PRONOSTIA 10.4. The coverage-driven gap is invariant to the temporal regime (minutes vs days).

All models actionable at 24h action window. The differentiator is confirmation density (trend visibility): the SLA-compliant deployment gives the operator a clear continuous trend; the SLA-violating one gives sparse confirmation points indistinguishable from sensor noise under typical alarm-management baselines (ISA-18.2).

## 5.3 · Cross-dataset validation

The alarm-density ratio is constant at 10–12× across PRONOSTIA accelerated bench (minutes-to-hours) and IMS realistic industrial timing (days), confirming that the gap is purely a function of coverage rate (SLA-vs-latency ratio), independent of degradation rate. The principle scales from accelerated test bench to industrial deployment.

# 6 · Phase 2 — Full DMCA lifecycle loop on bearing-replacement scenario

The Stage 2 admissibility validation above demonstrates ONE building block of DMCA on manufacturing data. To strengthen the manuscript and demonstrate the unique value of DMCA (closed-loop adaptive selection), we design a Phase 2 experiment that runs the full 5-stage loop on a realistic operational lifecycle: continuous monitoring of bearings, maintenance scheduling, hot model swap, and re-selection on a replaced asset. This is the primary new contribution proposed for the joint paper with IRT SystemX.

## 6.1 · Operational scenario

The simulation reproduces a real maintenance lifecycle:

Initial state: a bearing is monitored by an admissible DMCA model under Jetson Nano 50 ms SLA (e.g., PatchTST, latency-optimized for early-warning detection).

Degradation onset: bearing RMS rises. Stage 4 drift detector fires; Stage 2 admissibility raises the operational alarm. Maintenance window scheduled within action_window threshold.

Pre-maintenance model swap: in advance of the planned line stoppage, Stage 5 swaps to a steady-state monitoring model (e.g., MOMENT-L, baseline-stable for long-term healthy monitoring). The pre-swap is OTA during a permitted operational window.

Physical bearing replacement: operator + maintenance team replace the failing bearing during the scheduled stoppage. New bearing installed.

Post-replacement re-selection: new bearing presents a new signal baseline (lower healthy RMS, different σ, possibly different sensor calibration). Stage 1 re-profiles the asset context; Stage 2 re-runs TOPSIS over the model registry; the system selects the model best suited to the new asset baseline (potentially back to PatchTST, potentially a different choice if context changed).

Continuous loop: monitoring resumes, the cycle repeats across multiple bearings over the asset fleet lifetime.

## 6.2 · Technical implementation plan

| Component | What needs to be built | Owner |
| --- | --- | --- |
| Lifecycle data stitching | Concatenate PRONOSTIA bearing traces in sequence to simulate continuous monitoring across maintenance events; tag transitions | Lead author |
| Multi-model registry | Pre-benchmarked models with complementary profiles: detection-sensitive, monitoring-stable, fallback | Lead author (models already benchmarked in thesis) |
| Stage 4 drift typology extension | Extend v3.1.1 classifier to distinguish degradation-driven drift (no model swap, physical alarm) vs context-shift drift (model swap, no physical action). See §6.3. | Lead author (semantic refinement, key novelty) |
| Stage 5 hot-swap mechanics | Atomic model swap during permitted operational window; rollback on regression; shadow validation post-swap | Lead author |
| End-to-end loop orchestrator | Workflow integrating all stages over the stitched lifecycle data | Lead author |
| Evaluation harness | Metrics: total detection latency, swap latency, post-swap stabilization time, lifecycle MAE, alarm rate vs ISA-18.2 budget | Lead author |
| Distributed AAS substrate | Cloud/fog/edge submodel infrastructure on which lifecycle events and model-swap operations are exposed as DT service operations | IRT SystemX (Belfadel) |

Output: full DMCA loop empirically demonstrated on manufacturing lifecycle data, primary novel contribution of the joint paper. The lead author owns the framework end-to-end (Stages 2–5); the IRT SystemX contribution is bounded to the Stage 1 distributed AAS infrastructure on which the loop operates.

## 6.3 · Drift typology refinement: degradation vs context-shift

Currently the v3.1.1 drift classifier (thesis, Stage 4) distinguishes 7 drift types and applies a SWAP/NO_SWAP policy based on drift mode (ABRUPT/GRADUAL/INCREMENTAL/DISTRIBUTION_SHIFT → SWAP; VARIANCE_SHIFT/OUTLIER_DRIVEN → NO_SWAP). However, this policy does not yet distinguish between two semantically distinct causes:

| Observed pattern | Most likely cause | Correct DMCA response |
| --- | --- | --- |
| Gradual RMS rise from baseline toward failure threshold | Physical asset degradation (bearing failing) | Raise operational alarm; NO model swap (the model is doing its job); physical maintenance action by operator |
| Sharp RMS drop back to baseline after maintenance window | Asset replacement; new healthy bearing installed; context shifted | SWAP model (Stage 5); Stage 1 re-profile; Stage 2 re-select |
| RMS distribution shifts without crossing failure thresholds | Sensor recalibration, hardware upgrade, process change | SWAP model; no physical action |
| RMS variance grows without mean shift | Sensor degradation; signal-side issue | NO_SWAP; alert sensor maintenance team |

The refinement requires integrating a lifecycle-aware layer above the v3.1.1 classifier. Two implementation routes are open: (a) integration with external maintenance log to ground-truth replacement events; (b) heuristic discrimination based on temporal signature (gradual rise followed by abrupt drop = replacement event). The distributed AAS submodel infrastructure proposed by Belfadel is a natural substrate for tracking maintenance events as DT context signals.

## 6.4 · IRT SystemX collaboration scope

The Phase 2 lifecycle loop design above is led and executed by the lead author, continuing the methodological line of the thesis. The IRT SystemX collaboration scope is narrower and complementary: it is bounded to the Stage 1 distributed AAS infrastructure on which the loop operates.

Concrete contributions from IRT SystemX:

Distributed AAS architecture (§6 of the manuscript): cloud/fog/edge node design, submodel schema for runtime status (latency, load, drift indicators), TOPSIS-as-DT-service interface specification. This generalizes Stage 1 from the static JSON Phase 1 (thesis) to the distributed dynamic infrastructure required for multi-node deployments. This is the substrate on which lifecycle events become AAS submodel updates and model swap becomes a DT service operation.

Standards positioning (§2.2): mapping of framework components against IEC 63278 CDV, ISO 23247, ISO/IEC 30173 — area where Belfadel has committee-member expertise (Belfadel et al. PLM 2024, XRTwin4Industry).

Optional, high-leverage if available: an industrial-fleet bearing telemetry channel through SystemX partner relationships (Valeo, Renault, Safran, Airbus). Would enable a Case Study C industrial validation of the lifecycle loop, substantially strengthening the manuscript for IJAMT acceptance. To be discussed without forcing.

Co-authorship credibility for IJAMT review: a SystemX AAS-standards co-author improves acceptance probability in standards-aligned venues.

Out of scope for SystemX: the DMCA framework core (Stages 2–5), the drift classifier and its semantic refinement (§6.3), the lifecycle loop logic and orchestration, the missed-alarm simulation and experimental validation. These remain the lead author’s work, continuing the thesis methodological line.

# 7 · Open questions to align with Belfadel (2026-06-26 call)

Confirm §6 distributed-AAS extension scope: cloud/fog/edge submodels, TOPSIS-as-DT-service, IEC 63278 CDV / ISO 23247 conformance positioning. Any additional standards angle to integrate?

Phase 2 lifecycle loop (Section 6 of this document): is the scope realistic for the joint paper? Should the §6.3 drift-typology semantic refinement be a separate publication or integrated?

Industrial Case Study C: does IRT SystemX have a partner channel for real industrial bearing data (Valeo, Renault, Safran)? Even non-production telemetry on a limited time window would substantially strengthen the manuscript.

Framework programme: under which SystemX programme is the collaboration inquadrato (Confiance.ai legacy, European Trustworthy AI Association, internal initiative)? Implications on publication openness, IP framework, and France 2030 obligations.

Timeline alignment: any constraints from SystemX side on submission window or interim milestones to be aware of?

IP framework: SystemX standard template for external academic collaboration? Will be brought to TTO Polito (RIMIN / Area TRIN) for review before any binding commitment.

# 8 · Selected references (manufacturing PM stream added vs thesis)

Nectoux et al. 2012 — PRONOSTIA: An Experimental Platform for Bearings Accelerated Degradation Tests, IEEE PHM Challenge 2012.

Lee, Qiu, Yu, Lin 2007 — IMS Bearings Dataset, NASA Prognostics Data Repository.

ISO 10816-3:2009 / ISO 20816-3:2022 — Vibration severity standard for machine evaluation.

IEC 61508 / IEC 62061 — Safety Integrity Level (SIL) framework.

ISA-18.2 — Management of Alarm Systems for the Process Industries.

Lee, Lapira, Bagheri, Kao 2013 — Recent advances in predictive manufacturing systems, Manufacturing Letters.

Randall, Antoni 2011 — Rolling element bearing diagnostics — a tutorial, Mechanical Systems and Signal Processing.

Belfadel, Creff, Ben Hamida 2025 — Advancing Industrial Digital Twins Toward an Open Platform Aligned with Standards, IFIP AICT vol. 741.

Senseye / Siemens 2024 — True Cost of Downtime report.

Bifet & Gavalda 2007 — Learning from Time-Changing Data with Adaptive Windowing (ADWIN), SIAM Conference on Data Mining (reused from thesis).

# 9 · Status of this document and next steps

v0.3 reflects today experimental validation on PRONOSTIA + IMS (real data) and the new Phase 2 lifecycle loop design.

Stage 2 validation: complete. Real numbers available, two killer figures produced (Bearing 3_2 safety-critical flip; IMS Bearing 1 industrial timing).

Phase 2 full loop: design specified, implementation owned by lead author; primary novel methodological contribution proposed for the joint paper.

Drift typology semantic refinement (6.3): identified as key novelty, owned by lead author; open for Belfadel input on AAS-substrate integration.

IRT SystemX call 2026-06-26 will confirm distributed AAS scope and Section 6 author allocation to Belfadel; v0.4 will incorporate post-call decisions.